# Arc — Product Brief

# Overview

Arc is a desk lamp for people who work at the same desk for eight hours. It throws an even, flicker-free pool of light across a 900 by 600 millimetre work surface and dims warm, from 4000K at full brightness down to 2200K at its lowest setting.

It replaces the clip-on task lights and floor lamps our interviewees described as either too harsh or too dim, and it costs less than the architect lamps they admire but do not buy.

# Who it is for

Architects, writers, and engineers working from home or in small studios. They notice flicker, they care about colour, and they keep a tidy desk.

# Requirements

## Light quality

CRI 95 or better across the dimming range.

No visible flicker at any brightness; PWM above 20 kHz or constant-current dimming.

Warm dimming curve that tracks a filament from 4000K to 2200K.

## Mechanics

Two-section arm with friction hinges; holds position with the shade at full reach.

Shade diameter 140 mm, spun aluminium, matte interior.

Cast zinc base, 1.6 kg, with a felt underside.

## Controls

A single touch strip on the shade rim: tap for on and off, slide to dim.

Remembers the last brightness.

# Non-goals

App control or smart-home integration in version one.

Clamp mounting.

Battery operation.

# Open issues

Hinge torque drifts after 5,000 cycles in DVT; rework due October 17.

Driver EMI fails the conducted-emissions limit at 150 kHz by 3 dB.

Signed off by Priya Raman and Tom Okafor, September 28, 2026.